Autonomous Tractor Safety System

Objective 6.0 To find and display the DC motor position using LEDs in ATSS PCB board

TRISF DATA DIRECTION REGISTER F: 0x0000

| TRISF15 | TRISF14 | TRISF13 | TRISF12 | TRISF11 | TRISF10 | TRISF9 | TRISF8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
|  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  | Output |

| TRISF7 | TRISF6 | TRISF5 | TRISF4 | TRISF3 | TRISF2 | TRISF1 | TRISF0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 0 | 0 |  |  |  |  |  |  |
| Output | Output |  |  |  |  |  |  |

TRISA DATA DIRECTION REGISTER A: 0x0000

| TRISA15 | TRISA14 | TRISA13 | TRISA12 | TRISA11 | TRISA10 | TRISA9 | TRISA8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
|  | 0 |  |  |  |  |  |  |
|  | Output |  |  |  |  |  |  |

| TRISA7 | TRISA6 | TRISA5 | TRISA4 | TRISA3 | TRISA2 | TRISA1 | TRISA0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |

TRISB DATA DIRECTION REGISTER B: 0x3927

| TRISB15 | TRISB14 | TRISB13 | TRISB12 | TRISB11 | TRISB10 | TRISB9 | TRISB8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| OUTPUT | OUTPUT | INPUT | INPUT | INPUT | OUTPUT | OUTPUT | INPUT |

| TRISB7 | TRISB6 | TRISB5 | TRISB4 | TRISB3 | TRISB2 | TRISB1 | TRISB0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| OUTPUT | OUTPUT | INPUT | OUTPUT | OUTPUT | INPUT | INPUT | INPUT |

ANSB DATA DIRECTION REGISTER B: 0x0000

| ANSB15 | ANSB14 | ANSB13 | ANSB12 | ANSB11 | ANSB10 | ANSB9 | ANSB8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL |

| ANSB7 | ANSB6 | ANSB5 | ANSB4 | ANSB3 | ANSB2 | ANSB1 | ANSB0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL | DIGITAL |

LATF OUTPUT LATCH REGISTER F:  0x01C0

| LATF15 | LATF14 | LATF13 | LATF12 | LATF11 | LATF10 | LATF9 | LATF8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
|  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  | LED1 off |

| LATF7 | LATF6 | LATF5 | LATF4 | LATF3 | LATF2 | LATF1 | LATF0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
| 1 | 1 |  |  |  |  |  |  |
| LED2 off | LED3 off |  |  |  |  |  |  |

LATA OUTPUT LATCH REGISTER A:  0x4000

| LATAF15 | LATA14 | LATA13 | LATA12 | LATA11 | LATA10 | LATA9 | LATA8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |
|  | 1 |  |  |  |  |  |  |
|  | LED4 off |  |  |  |  |  |  |

| LATA7 | LATA6 | LATA5 | LATA4 | LATA3 | LATA2 | LATA1 | LATA0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| I/O | I/O | I/O | I/O | I/O | I/O | I/O | I/O |

INTCON2 INTERRUPT CONTROL REGISTER 2: 0x0000

| ALTIVT | DISI | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ALT INT VECTOR TABLE EN | DISI INSTR STATUS |  |  |  |  |  |  |

| - | - | - | INT4EP | INT3EP | INT2EP | INT1EP | INT0EP |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | EXT INT4 POLARITY | EXT INT3 POLARITY | EXT INT2 POLARITY | EXT INT1 POLARITY |  |
|  |  |  | 0 | 0 | 0 | 0 |  |
|  |  |  | +VE EDGE | +VE EDGE | +VE EDGE | +VE EDGE |  |

IFS1 INTERRUPT FLAG STATUS REGISTER 1: 0x0000

| U2TXIF | U2RXIF | INT2IF | T5IF | T4IF | OC4IF | OC3IF | DMA2IF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UART2 Tx REQUEST | UART2 Rx REQUEST | EXT INT2 REQUEST | TMR4 INT REQUEST | TMR4 INT REQUEST | O/P CMP CH4 INT | O/P CMP CH3 INT | DMA CH2 INT |
|  |  | 0 |  |  |  |  |  |
|  |  | NO INT |  |  |  |  |  |

| - | IC7IF | - | INT1IF | CNIF | CMIF | MI2C1IF | S12C1IF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I/P CH 7 CAPTURE |  | EXT INT1 REQUEST | I/P CHNG NOTIF INT | CMP INT | MASTER I2C1 INT | SLAVE I2C1 INT |
|  |  |  | 0 |  |  |  |  |
|  |  |  | NO INT |  |  |  |  |

IFS3 INTERRUPT FLAG STATUS REGISTER 3: 0x0000

| - | RTCIF | DMA5IF | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RTC INT REQ | DMA CH5 INT |  |  |  |  |  |

| - | INT4IF | INT3IF | - | - | MI2C2IF | SI2C2IF | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EXT INT4 REQUEST | EXT INT3 REQUEST |  |  | MASTER I2C2 INT | SLAVE I2C2 INT |  |
|  | 0 | 0 |  |  |  |  |  |
|  | NO INT | NO INT |  |  |  |  |  |

IEC1 INTERRUPT ENABLE CONTROL REGISTER 1: 0x2010

| U2TXIE | U2RXIE | INT2IE | T5IE | T4IE | OC4IE | OC3IE | DMA2IE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UART2 Tx REQUEST | UART2 Rx REQUEST | EXT INT2 REQUEST | TMR4 INT REQUEST | TMR4 INT REQUEST | O/P CMP CH4 INT | O/P CMP CH3 INT | DMA CH2 INT |
|  |  | 1 |  |  |  |  |  |
|  |  | ENABLE |  |  |  |  |  |

| - | IC7IE | - | INT1IE | CNIE | CMIE | MI2C1IE | S12C1IE |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | I/P CH 7 CAPTURE |  | EXT INT1 REQUEST | I/P CHNG NOTIF INT | CMP INT | MASTER I2C1 INT | SLAVE I2C1 INT |
|  |  |  | 1 |  |  |  |  |
|  |  |  | ENABLE |  |  |  |  |

IEC3 INTERRUPT ENABLE CONTROL REGISTER 3: 0x0060

| - | RTCIE | DMA5IE | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RTC INT REQ | DMA CH5 INT |  |  |  |  |  |

| - | INT4IE | INT3IE | - | - | MI2C2IE | SI2C2IE | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EXT INT4 REQUEST | EXT INT3 REQUEST |  |  | MASTER I2C2 INT | SLAVE I2C2 INT |  |
|  | 1 | 1 |  |  |  |  |  |
|  | ENABLE | ENABLE |  |  |  |  |  |

IPC5 INTERRUPT PRIORITY CONTROL REGISTER 5: 0x0004

| - | - | - | - | - | IC7IP2 | IC7IP1 | IC7IP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | I/P CAPTURE CH7 PRIORITY |  |  |

| - | - | - | - | - | INT1IP2 | INT1IP1 | INT1IP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EXT INT1 PRIORITY |  |  |
|  |  |  |  |  | 1 | 0 | 0 |
|  |  |  |  |  | 4 |  |  |

IPC7 INTERRUPT PRIORITY CONTROL REGISTER 7: 0x0040

| - | U2TXIP2 | U2TXIP1 | U2TXIP0 | - | U2RXIP2 | U2RXIP1 | U2RXIP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | UART2 TX PRIORITY |  |  |  | UART2 RX PRIORITY |  |  |

| - | INT2IP2 | INT2IP1 | INT2IP0 | - | T5IP2 | T5IP1 | T5IP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EXT INT2 PRIORITY |  |  |  | TMR5 INT PRIORITY |  |  |
|  | 1 | 0 | 0 |  |  |  |  |
|  | 4 |  |  |  |  |  |  |

IPC13 INTERRUPT PRIORITY CONTROL REGISTER 13: 0x0440

| - | - | - | - | - | INT4IP2 | INT4IP1 | INT4IP0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EXT INT4 PRIORITY |  |  |
|  |  |  |  |  | 1 | 0 | 0 |
|  |  |  |  |  | 4 |  |  |

| - | INT3IP2 | INT3IP1 | INT3IP0 | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EXT INT3 PRIORITY |  |  |  |  |  |  |
|  | 1 | 0 | 0 |  |  |  |  |
|  | 4 |  |  |  |  |  |  |

RPINR0 PERIPHERAL PIN SELECT INPUT REGISTER 0: 0x2700

| - | - | INT1R5 | INT1R4 | INT1R3 | INT1R2 | INT1R1 | INT1R0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ASSIGN EXT INT1 CORRESPONDING TO PINS IN RP0 |  |  |  |  |  |
|  |  | 1 | 0 | 0 | 1 | 1 | 1 |
|  |  | PIN 39 IS HALL SENSOR1 |  |  |  |  |  |

| - | - | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |

RPINR1 PERIPHERAL PIN SELECT INPUT REGISTER 1: 0x2928

| - | - | INT3R5 | INT3R4 | INT3R3 | INT3R2 | INT3R1 | INT3R0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ASSIGN EXT INT3 CORRESPONDING TO PINS IN RP1 |  |  |  |  |  |
|  |  | 1 | 0 | 1 | 0 | 0 | 1 |
|  |  | PIN 41 IS HALL SENSOR3 |  |  |  |  |  |

| - | - | INT2R5 | INT2R4 | INT2R3 | INT2R2 | INT2R1 | INT2R0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ASSIGN EXT INT2 CORRESPONDING TO PINS IN RP1 |  |  |  |  |  |
|  |  | 1 | 0 | 1 | 0 | 0 | 0 |
|  |  | PIN 40 IS HALL SENSOR2 |  |  |  |  |  |

RPINR2 PERIPHERAL PIN SELECT INPUT REGISTER 2: 0x002A

| - | - | - | - | - | - | - | - |
| --- | --- | --- | --- | --- | --- | --- | --- |

| - | - | INT4R5 | INT4R4 | INT4R3 | INT4R2 | INT4R1 | INT4R0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ASSIGN EXT INT2 CORRESPONDING TO PINS IN RP1 |  |  |  |  |  |
|  |  | 1 | 0 | 1 | 0 | 1 | 0 |
|  |  | PIN 42 IS HALL SENSOR4 |  |  |  |  |  |

BTN8982A Motor driver

Rotary encoder output of a DC motor rotating clockwise

Tabulate the voltage levels of A and B alternatively every qardature starting with the first rising edge in ENC A, to get five distinct patterns that repeat themselves – 1110, 1000, 0111, 0111.

Rotary encoder output of a DC motor rotating anticlockwise

Tabulate the voltage levels of A and B alternatively every qardature starting with the first rising edge in ENC A, to get five distinct patterns that repeat themselves – 10111, 1101, 0100, 0010.

Hall Effect sensor

A rotary encoder, also called a shaft encoder, is an electro-mechanical device that converts the angular position or motion of a shaft or axle to analog or digital output signals.

Rotary encoders measure rotation of a shaft, while linear encoders measure distance traveled. For both types of encoder, the position measurement can be either incremental or absolute. An incremental encoder measures change in position, but does not keep track of actual position. When power is interrupted, incremental encoders lose their position reference and must start over via a re-homing sequence to a reference point.

Incremental encoders work by producing a specific number of equally spaced pulses per revolution (PPR) or per distance (PPM—pulses per millimeter, or PPI—pulses per inch). When one set of pulses, or output channel, is used, the encoder can determine position only.  But most incremental encoders use quadrature output, which consists of two channels, typically referred to as channel A and channel B, that are out of phase by 90 degrees. Quadrature output allows the encoder to also sense direction, by determining which channel is leading and which is following.

With quadrature output, three types of encoding can be used: X1, X2, or X4. The difference between these encoding types is simply which edges of which channel are counted during movement, but their influence on encoder resolution is significant.

With X1 encoding, either the rising (aka leading) or the falling (aka following) edge of channel A is counted. If channel A leads channel B, the rising edge is counted, and the movement is forward, or clockwise. Conversely, if channel B leads channel A, the falling edge is counted, and the movement is backwards, or counterclockwise.

In this function,we can see that if encoder B/channel B is 0 and the encoder A also 0.The pulses will count in negative, that’s mean the motor is moving counter clockwise.The encoder B is lagging the encoder A. To conclude, when the channel B is leading the channel A means the motor/encoder is moving counter clockwise.When the channel A is leading the channel B means the motor/encoder is moving clockwise.And the pulses will count when channel A is trigger to 1.

X1 encoding, in which either the rising or the falling edge of channel A is counted. Notice that when channel B is leading, the movement is considered counterclockwise or reverse, and the count is decreased. For rotary encoders, position is calculated by dividing the number of edges counted by the product of the number of pulses per revolution and the encoding type described above (1, 2, or 4), and then multiplying the result by 360 in order to get degrees of motion.

degrees = edgeCount * 360 /(x * N), x = type of encoding (X1, X2, X4)

The encoder has a disk with evenly spaced contact zones that are connected to the common pin C and two other separate contact pins A and B, as illustrated below.

When the disk will start rotating step by step, the pins A and B will start making contact with the common pin and the two square wave output signals will be generated accordingly.

In a simple DC motor there are two main components, the “stator” and the “armature”. The stator is a permanent magnet and provides a constant magnetic field. The armature, which is the rotating part, is a simple coil.

The armature is connected to a DC power source using a 2-piece ring installed around the motor shaft, these ring sections are called “commutator rings”. The two pieces of the commutator rings are connected to each end of the armature coil. Direct Current of a suitable voltage is applied to the commutator rings via two “brushes” that rub against the rings.

When DC is applied to the commutator rings it flows through the armature coil, producing a magnetic field. This field is attracted to the stator magnet (remember, opposite magnetic polarities attract, similar ones repel) and the motor shaft begins to spin.

The motor shaft rotates until it arrives at the junction between the two halves of the commutator. At that point the brushes come into contact with the other half of the commutator rings, reversing the polarity of the armature coil (or coils, most modern DC motors have several). This is great because at this point the motor shaft has rotated 180 degrees and the magnetic field polarities need to be reversed for the motor to continue rotating. This process repeats itself indefinitely until the current is removed from the armature coils.

To reverse the direction in which the DC motor rotates you simply reverse the polarity of the DC current that you apply to it. One method of changing the speed of a DC motor is to simply reduce its supply voltage. While this will work to some degree it is actually not a very good method of controlling motor speed as lowering the voltage will also lower the torque that the motor is capable of producing. Also, once the voltage drops below a certain point the motor will not rotate at all.

With PWM the motor is sent a series of pulses. Each pulse is of the full voltage that the motor can handle so a 6-volt motor will be sent 6-volt pulses while a 12-volt motor will be sent 12 volt pulses. The width of the pulses are varied to control the motor speed, pulses with a narrow width will cause the motor to spin quite slowly. Increasing the pulse width will increase the speed of the motor, as illustrated below.

In order to stop the motor completely you just stop pulsing it, essentially sending it zero volts. To run it at full speed you send it the full voltage, again without pulsing it.

An “H-Bridge” is simply an arrangement of switching the polarity of the voltage applied to a DC motor, thus controlling its direction of rotation.

In the first diagram we can see four switches which are all in the open or “off” position. In the center of the circuit is a DC motor. If you look at the circuit as it is drawn here you can distinctly see a letter “H”, with the motor attached in the center or “bridge” section – thus the term “H-Bridge”.

Now we’ll open those switches and close the other two. As you can see this causes the polarity of the voltage applied to the motor to be reversed, resulting in our motor spinning counterclockwise.

The BTN8982A actually contains two complete H-Bridge circuits, so it is capable of driving a pair of DC motors.

The board has four input terminals plus two enable terminals. You will use these terminals to control both direction and speed or each motor. They are as follows:

- IN1 – Input 1 for Motor A

- IN2 – Input 2 for Motor A

- IN3 – Input 3 for Motor B

- IN4 – Input 4 for Motor B

- INH1 – Inhibit line for Motor A (SLEEP MODE)

- INH2 – Inhibit Line for Motor B (SLEEP MODE)

The INHIBIT line can be used to turn the motor on, to turn it off and to control its speed. When the Enable line is at 5 Volts (1) the motor will be on. Grounding the INHIBIT line (0) will turn the motor off. To control the speed of the motor you apply a Pulse Width Modulation (PWM) signal to the INHIBIT line. The shorter the pulse width, the slower the motor will spin.

Truth table

Code Prg06.c

/*************************************************************************************

Source code file:      Prg06.c

Author, version, date: PSS ver 1.0 01.11.24

Program function:      Motor position

Simulation:            PIC24FJ128GA308 MCU, MPLAB X IDE ver 6.05, XC16 ver 2.10

*************************************************************************************/

/*

SW1 switch on safety box for running (LED ON) the clutch (LED1) and brake (LED3) motors or stopping the motors (LED OFF)

SW2 switch near driver for running the clutch (LED2) and brake (LED4) motors in a clockwise (LED ON) or anticlockwise direction (LED OFF)

LED1: ON - clutch motor is running,       OFF - clutch motor is not running

LED2: ON - clutch motor is fully engaged, OFF - clutch motor is fully disengaged

LED3: ON - brake  motor is running,       OFF - brake  motor is not running

LED4: ON - brake  motor is fully engaged, OFF - brake  motor is fully disengaged

*/

#include <xc.h>

#include <libpic30.h>

#define LED1 LATFbits.LATF8

#define LED2 LATFbits.LATF7

#define LED3 LATFbits.LATF6

#define LED4 LATAbits.LATA14

#define spareOnOffSwitch LATDbits.LATD15

#define actuatorReleaseSwitch LATDbits.LATD14

#define INH1 LATBbits.LATB4				//  Motor driver inhibit pins

#define INH2 LATBbits.LATB6

#define INH3 LATBbits.LATB9

#define INH4 LATBbits.LATB14

#define IN1  LATBbits.LATB3				//  Motor input pins

#define IN2  LATBbits.LATB7

#define IN3  LATBbits.LATB10

#define IN4  LATBbits.LATB15

#define IS1  LATBbits.LATB5				//  Motor current sensor pins

#define IS2  LATBbits.LATB2

#define IS3  LATBbits.LATB8

#define IS4  LATBbits.LATB11

#define ON         	       1

#define OFF                0

#define BEGINNING      	0				//  Fully disengaged

#define MIDDLE            -1 				//  Fifty percent engaged

#define END        	       1				//  Fully engaged

#define hallSensor1 LATFbits.LATF4

#define hallSensor2 LATFbits.LATF5

#define hallSensor3 LATFbits.LATF3

#define hallSensor4 LATFbits.LATF2

uint8_t spareOnOffSwitchNow;

uint8_t actuatorReleaseSwitchNow;

uint8_t motorsState;

uint8_t motorsStateNow;

uint8_t clutchMotorState;

uint8_t clutchMotorStateNow;

uint8_t clutchMotorPositionNow;

uint8_t bit0;

uint8_t bit1;

uint8_t bits10;

uint8_t bits32;

uint8_t bits3210;

uint8_t brakeMotorState;

uint8_t brakeMotorStateNow;

uint8_t brakeMotorPositionNow;

uint8_t motorPosition;

uint16_t motorDegrees;

uint8_t motorPulsesCount;

uint8_t motorPulsesPerRevolution = 280;

uint8_t INH1Now;

uint8_t INH2Now;

uint8_t INH3Now;

uint8_t INH4Now;

uint8_t IN1Now;

uint8_t IN2Now;

uint8_t IN3Now;

uint8_t IN4Now;

uint8_t extInt1;

uint8_t extInt2;

uint8_t extInt3;

uint8_t extInt4;

void initializeMotors(void);

uint8_t turnOnMotors(void);

uint8_t getClutchMotorState(void);

uint8_t getBrakeMotorState(void);

uint8_t getMotorPosition();

void engageClutchMotor(void);

void disEngageClutchMotor(void);

void engageBrakeMotor(void);

void disEngageBrakeMotor(void);

int main()

{

initializeMotors();

turnOnMotors();

while (1)

{

clutchMotorStateNow = getClutchMotorState();

if (clutchMotorStateNow)

if (!IS1 & !IS2)		//  No current sensor error

clutchMotorPositionNow = getMotorPosition();

brakeMotorStateNow  = getBrakeMotorState();

if (brakeMotorStateNow)

if (!IS3 & !IS4)

brakeMotorPositionNow   = getMotorPosition();

if (clutchMotorStateNow)

LATFbits.LATF6 = 0;	//  LED3 RF6 ON, clutch motor is running

else

LATFbits.LATF6 = 1; 	//  LED3 RF6 OFF, clutch motor is not running

if (clutchMotorPositionNow)

LATFbits.LATF7 = 0;	//  LED2 RF7 ON, clutch motor is fully engaged

else

LATFbits.LATF7  = 1;	//  LED2 RF7 OFF, clutch motor is fully disengaged

if (brakeMotorStateNow)

LATFbits.LATF8 = 0;	//  LED1 RF8 ON, brake motor is running

else

LATFbits.LATF8 = 1;	//  LED1 RF8 OFF, brake motor is not running

if (brakeMotorPositionNow)

LATAbits.LATA14 = 0;	//  LED4 RA14 ON, brake motor is fully engaged

else

LATAbits.LATA14 = 1;	//  LED4 RA14 OFF, brake motor is fully disengaged

}

}

void initializeMotors(void)

{

ANSB  = 0x0000;			//  Digital mode

TRISA = 0x0000;			//  Output mode(O):0, Input mode(I):1, RA14:LED4(O)

TRISB = 0x3927;			//  RB0:PGEC(I), RB1:PGED(I), RB2:IS2(I), RB3:IN1(O),

//  RB4:INH1(O), RB5:IS1(I), RB6:INH2(O), RB7:IN2(O), RB8:IS3(I), RB9:INH3(O),

//  RB10:IN3(O), RB11:IS4(I), RB12:CurrentSensorInput(I), RB13:RadarSensorInput(I),

//  RB14:INH4(O), RB15:IN4(O)

TRISF = 0x003C;			//  RF2:HallSensor4(I), RF3:HallSensor3(I),

//  RF4:HallSensor1(I), RF5:HallSensor2(I)

TRISD = 0xC110;			//  RD0:CAN CS(O), RD4:JetsonXavierTx(I),

//  RD5:JetsonXavierRx(O), RD8:CAN SDI(I), RD9:CAN SDO(O), RD10:CAN SCL(I), RD11:CAN

//  INT(O), RD14:spareOnOffSwitch(I), RD15:actuatorReleaseSwitch(I)

LATF = 0x01C0;			//  Set all LEDs LOW initially

LATA = 0x4000;			//  All the LEDS are in common cathode configuration

PORTA = PORTB = PORTD = 0x0000;

LATA  = LATB  = LATD  = 0x0000;

RPINR0    = 0x2700;//  PPS Peripheral Pin Select INT1(39:0x27)-hallSensor1(39)

RPINR1    = 0x2928;//  INT3(41:0x29)-hallSensor3(41) INT2(40:0x28)-hallSensor2(40)

RPINR2    = 0x002A;//  INT4(42:0x2A)-hallSensor4(42)

INTCON2   = 0x0000;		//  INT1-4 are +ve edge triggered

IFS1	     = 0x0000;	//  Clear INT1,2 flags

IFS3      = 0x0000;		//  Clear INT3,4 flags

IEC1      = 0x2010;		//  Enable INT1,2

IEC3      = 0x0060;		//  Enable INT3,4

IPC5      = 0x0004;		//  Set INT1 priority as 4 (default)

IPC7      = 0x0040;		//  Set INT2 priority as 4 (default)

IPC13     = 0x0440;		//  Set INT3,4 priority as 4 (default)

LED1    = 1;			//  Set all LED OFF initially

LED2    = 1;

LED3    = 1;

LED4    = 1;

clutchMotorPositionNow = BEGINNING;

brakeMotorPositionNow  = BEGINNING;

}

uint8_t turnOnMotors(void)

{

spareOnOffSwitchNow      = spareOnOffSwitch;

actuatorReleaseSwitchNow = actuatorReleaseSwitch;

if (spareOnOffSwitchNow)	//  RD15 Spare ON/OFF switch

{

if (actuatorReleaseSwitchNow)	//  RD14 actuatorReleaseSwitch

{

while (clutchMotorPositionNow != END)

{

engageClutchMotor();	//  Motors run in clockwise direction

if (!IS1 & !IS2)

clutchMotorPositionNow   = getMotorPosition();

}

while (brakeMotorPositionNow != END)

{

engageBrakeMotor();

if (!IS3 & !IS4)

brakeMotorPositionNow   = getMotorPosition();

}

}

else

{

while (clutchMotorPositionNow != BEGINNING)

{

disEngageClutchMotor();	//  Motors run in anticlockwise direction

if (!IS1 & !IS2)

clutchMotorPositionNow   = getMotorPosition();

}

while (brakeMotorPositionNow != BEGINNING)

{

disEngageBrakeMotor();

if (!IS3 & !IS4)

brakeMotorPositionNow   = getMotorPosition();

}

}

}

else

{

INH1 = 0;			//  Clutch motor in SLEEP inactive mode

INH2 = 0;

INH3 = 0;			//  Brake motor in SLEEP inactive mode

INH4 = 0;

clutchMotorStateNow = OFF;

brakeMotorStateNow  = OFF;

}

}

uint8_t getClutchMotorState(void)

{

INH1Now = INH1;

INH2Now = INH2;			//  RB4 is INH1, RB6 is INH2

IN1Now  = IN1;			//  RB3 is IN1, RB7 is IN2

IN2Now  = IN2;

if ((INH1Now) & (INH2Now)) 	//  Low INHIBIT puts BT8982A in SLEEP mode, clutch

//  motor is in an OFF state

{

if (((IN1Now & (!IN2Now)) | ((!IN1Now) & IN2Now))) //  (IN1, IN2) = (0, 1) is

//  CLOCKWISE, (IN1, IN2) = (1, 0) is ANTICLOCKWISE

clutchMotorState = ON;

else

clutchMotorState = OFF;

}

return clutchMotorState;

}

uint8_t getBrakeMotorState(void)

{

INH3Now = INH3;			//  RB10 is INH3, RB15 is INH4

INH4Now = INH4;

IN3Now  = IN3;			//  RB9 is IN3, RB14 is IN4

IN4Now  = IN4;

if ((INH3Now) & (INH4Now))    //  Low INHIBIT puts BT8982A in SLEEP mode, brake

//  motor is in an OFF state

{

if (((IN3 & (!IN4)) | ((!IN3) & IN4))) //  (IN3, IN4) = (0, 1) is CLOCKWISE,

//  (IN3, IN4) = (1, 0) is ANTICLOCKWISE

brakeMotorState = ON;

else

brakeMotorState = OFF;

}

return brakeMotorState;

}

uint8_t getMotorPosition()

{

if ((extInt1 & extInt2) | (extInt3 & extInt4))

{

bit1 = hallSensor3;

bit0 = hallSensor4;

bits10 = (bit1 << 1) | bit0;

bits3210  = (bits32 << 2) | bits10;

if (bits3210 == 0b1101 || bits3210 == 0b0100 || bits3210 == 0b0010 ||

bits3210 == 0b1011) motorPulsesCount--;

if (bits3210 == 0b1110 || bits3210 == 0b0111 || bits3210 == 0b0001 ||

bits3210 == 0b1000) motorPulsesCount++;

bits32 = bits10;

motorDegrees = motorPulsesCount * 360 / motorPulsesPerRevolution;	//  degrees =

//  edgeCount * 360 /(x * N), x = type of encoding (X1, X2, X4), x = 1 for X1

switch (motorDegrees)	//  N is the pulse count for one revolution

{

case 0:

motorPosition = BEGINNING;

break;

case 180:

motorPosition = MIDDLE;

break;

case 360:

motorPosition = END;

break;

}

}

return motorPosition;

}

void engageClutchMotor(void)

{

INH1 = 1;

INH2 = 1;

IN1  = 1;

IN2  = 0;				//  Clockwise motor

}

void disEngageClutchMotor(void)

{

INH1 = 1;

INH2 = 1;                     //  Counterclockwise motor

IN1  = 0;

IN2  = 1;

}

void engageBrakeMotor(void)

{

INH3 = 1;

INH4 = 1;

IN3  = 1;

IN4  = 0;

}

void disEngageBrakeMotor(void)

{

INH3 = 1;

INH4 = 1;

IN3  = 0;

IN4  = 1;

}

void  __attribute__((interrupt, no_auto_psv)) _INT1Interrupt(void)

//  External Interrupt 1 service sub routine

{

IFS1bits.INT1IF = 0;		//  Clear External Interrupt Flag INT1IF(0)

extInt1         = 1;

}

void  __attribute__((interrupt, no_auto_psv)) _INT2Interrupt(void)

//  External Interrupt 2 service sub routine

{

IFS1bits.INT2IF = 0;		//  Clear External Interrupt Flag INT2IF(0)

extInt2         = 1;

}

void  __attribute__((interrupt, no_auto_psv)) _INT3Interrupt(void)

//  External Interrupt 3 service sub routine

{

IFS3bits.INT3IF = 0;		//  Clear External Interrupt Flag INT3IF(0)

extInt3         = 1;

}

void  __attribute__((interrupt, no_auto_psv)) _INT4Interrupt(void)

//  External Interrupt 4 service sub routine

{

IFS3bits.INT4IF = 0;		//  Clear External Interrupt Flag INT4IF(0)

extInt4         = 1;

}